Metric of Comparison

To evaluate the efficiency of proposed algorithms, the following metrics are taken into consideration in all simulations.

- Network Lifetime: The network lifetime is defined as the time in which the first node is died. Standard deviation and average values for network lifetime are computed in all scenarios.

- Residual Energy: The battery level of sensor nodes at the end of simulation.

- Number of Nodes Alive: A uniform distribution of energy consumption across the network can increase the number of nodes alive at the end of simulation.

- End-to-end Delay: This metric indicates the latency of packet delivery from the source node to the sink. Standard deviation and average values for delay are computed in all scenarios.

- Delivery Ratio: Delivery ratio is defined as the number of packets successfully reached the sink nodes.

- Network Load: The network load is computed as the total energy consumption divided by the total number of packet reported.

- Energy Consumption: This metric shows the amount of energy used during the simulation time. It is calculated by subtraction of residual energy from initial power level of sensor nodes.

- Packet loss: Number of lost packets due to node failure.

Simulation Setup and Network Configuration

NS2 (ver. 2.33) as an evaluation framework is used in this thesis. It is employed for all simulation scenarios in order to evaluate all metrics of comparison.

| Parameter | Value |
| --- | --- |
| Network simulator | NS2 (ver 2.33) |
| Network topology | Grid |
| The number of nodes | 7×7, 11×11, 15×15, 19×19, 23×23 |
| The number of sources | 3, 4, 5 |
| Field size | 2000 m by 2000 m |
| Maximum transmit range | 75 m |
| Sensing range | 75 m |
| Physical and MAC layer | IEEE 802.11 |
| Energy/bit used by transmitter (λ11) | 50 nJ/bit |
| Energy/bit used by transmitter amplifier (λ2) | 0.0013 pJ/bit/m4 |
| Energy/bit used by receiver (λ12) | 50 nJ/bit |
| Packet size | 100 bytes |
| Packet rate | 1 packet every 6 seconds |
| Number of continues packets per event | 10, 30 packets |
| Initial energy | 0.25 joules |
| Energy-threshold (for dead nodes) | 0.0025 joules (1% of initial energy) |
| Sink velocity | 20 m/s |
| Experiments iteration | 100 times |
| Sink repositioning interval | Per 6 seconds |